First find out your IP number of your Router…

-you may have to do a firmware reset & on the bottom of this router is the default admin user & password code…

-if you are connected you can see from the network properties the IPv4 DNS Server address…

Below is done in Edge or Chrome browser…where your ipv4 DNS Server is the number entered.

Username: admin (by default) Password: you set (or bottom of router has a printed pwd; if you do a firmware reset)

See there are 4 clients listed, the SSID, and WPA-PSK key (you set up…)

WiFi Setup: Asus RT-N300 B1 Router

Pick the Administration tab….

It allows 1)Wireless Access Point	2)Router	or 	3)Repeater

SEE ABOVE “AP MODE” IS SETUP!!!

Set to default, Open browser url    http://router.asus.com

http://www.asusrouter.com   this one works feb2-2024

Notes:

-setup as Wireless Access Point  cable connect to firewalled Jack in room.

-ensure desktop computers on same firewalled router, if a laptop is used you must connect to this network; not Dpcdsb!!!

-one instance on your computer has Thonny (local on bottom right of app set)

-2nd instance on your computer has Thonny(micropython on bottom right of app set)

-you start your pico-W program and it goes from waiting to getting an IP number

-you go to local Thonny app, start the client RGB program, type a colour

-if connected the PICO-W lcd will show the message is received from the client over the IP….

Note: Internet setup …

2nd Option Asus Router Discovery Tool:

Use Asus  “Device Discovery” program…