# If Statements Worksheet

Find errors:       int x = 15;

1.	if x<3  {                     (1 error)

Serial.println ("apple");

}

2.     if (x=5)  {		 			(1 error)

Serial.println ("apple");

}

3. if (x=5)		   (1 error)

Serial.println ("I like");

Serial.println ("apples");

else

Serial.println ("I don’t like apples");

4. 		 			           (2 errors)

if (x>5  && 5!=3)  {

Serial.println ("I like");

Serial.println ("apples");

}

else if (x<5  && 5!=3){

Serial.println ("I don’t like apples");

}

else (x==5)

Serial.println ("I don’t know if I like apples")

5. What will be printed to the serial monitor?

int sensorValue = 600;

void setup() {

Serial.begin(9600);

}

void loop() {

if (sensorValue > 500) {

Serial.println("High value");

}

}

6.  What will happen?

int buttonState = HIGH;

void setup() {

pinMode(13, OUTPUT);

}

void loop() {

if (buttonState == HIGH) {

digitalWrite(13, HIGH);

}

}

7. What will be printed to the serial monitor?

int temperature = 20;

void setup() {

Serial.begin(9600);

}

void loop() {

if (temperature < 25) {

Serial.println("Temperature is low");

}

}

8. What will be printed to the serial monitor?

int switchState = LOW;

void setup() {

Serial.begin(9600);

}

void loop() {

if (switchState == LOW) {

Serial.println("Switch is off");

}

}

9.  What will happen?

int lightLevel = 800;

void setup() {

pinMode(8, OUTPUT);

pinMode(7, OUTPUT);

pinMode(5, OUTPUT);

}

void loop() {

if (lightLevel > 900) {

digitalWrite(7,  HIGH);

} else if (lightLevel > 700) {

digitalWrite(8, HIGH);

} else {

digitalWrite(5, HIGH);

}

}

State the output:

int MAX = 25;

int num1 = 12;

10.	if (num1 < MAX)

Serial.println("apple");

11.	if (num1 < 10){

Serial.println("apple");

}

else{

Serial.println("orange");

}

12.	if (num1 <= 12){

Serial.println("apple");

}

else{

Serial.println("orange");

}

13.	if (num1 < MAX){

if (MAX >= 7){

Serial.println("apple");

}

}

Serial.println("orange");

14.	if (num1 == MAX){

if (num1 == 12)

Serial.println("apple");

}

Serial.println("orange");

15.	if (num1 <= 18){

if (num1 < 0)

Serial.println("apple");

else

Serial.println("orange");

}

Serial.println("pear");

16.	if (MAX == 25)

Serial.println("apple");

if (num1<=12)

Serial.println("orange");

if(num1==12 && 3>1 || num1==2)

Serial.println("pear");

For exercises 17 to 19, write code segments that will perform the specified action.

Assume that all variables have already been declared and given values.

17.    Print "Hurrah!" if the variable my_value is positive.

18.	Increment the integer variable total if total is zero and decrement total otherwise.  (Hint: 'x++' will increment the value of 'x', 'x--' will decrement the value of 'x').

19.	Print "num is zero", "num is negative", or "num is positive" as appropriate based on the current value of the variable num.